# Experiment 2.1

Student Name: Nabha Varshney					UID: 20BCS4995

Branch: B.E.-CSE			 	                          	Section/Group: 20BCS704-A

Semester: 6th								Date of Performance: 04/04/2023

Subject Name: Internet of Things Lab				Subject Code: 20CSP-358

- Aim:

# To measure the distance of an object using an ultrasonic sensor.

- Objective:

- Learn about interfacing.

- Learn about IoT programming.

- Requirements:

- Arduino Uno R3 board

- Ultrasonic sensor (HC-SR04)

- 16×2 LCD I2C Display

- Jumper Wires

- Procedure:

Arduino:

It is an open-source electronics platform. It consists ATmega328 8-bit Micro controller. It can be able to read inputs from different sensors & we can send instructions to the micro controller in the Arduino. It provides Arduino IDE to write code & connect the hardware devices like Arduino boards & sensors.

Ultrasonic Sensor:

An ultrasonic Sensor is a device used to measure the distance between the sensor and an object without physical contact.This device works based on time-to-distance conversion.

##### Setup

- Connect the Echo pin of the sensor to the D2 pin of the Arduino.

- Connect the Trig pin of the sensor to the D3 pin of the Arduino.

- Navigate to Tools and select board and port.

- Verify and compile the code, then upload the code to the Arduino Uno R3 board.

- Monitor the output in the Serial monitor (Set the baud rate as 9600). To open Serial monitor Tools>Serial Monitor or (Ctrl+Shift+M).

- Code/Program:

const int pingPin = 3; // Trigger Pin of Ultrasonic Sensor

const int echoPin = 2; // Echo Pin of Ultrasonic Sensor

const int led = 13;

void setup() {

Serial.begin(9600); // Starting Serial Terminal

pinMode(led,OUTPUT);

pinMode(8, OUTPUT);

}

void loop() {

long duration, inches, cm;

pinMode(pingPin, OUTPUT);

digitalWrite(pingPin, LOW);

delayMicroseconds(2);

digitalWrite(pingPin, HIGH);

delayMicroseconds(10);

digitalWrite(pingPin, LOW);

pinMode(echoPin, INPUT);

duration = pulseIn(echoPin, HIGH);

inches = microsecondsToInches(duration);

cm = microsecondsToCentimeters(duration);

Serial.print(inches);

Serial.print("in, ");

Serial.print(cm);

Serial.print("cm");

Serial.println();

if(cm <50)

{

digitalWrite(led, HIGH);

digitalWrite(8,HIGH);

}

else

{

digitalWrite(led, LOW);

digitalWrite(8,LOW);

}

delay(1000);

}

long microsecondsToInches(long microseconds) {

return microseconds / 74 / 2;

}

long microsecondsToCentimeters(long microseconds) {

return microseconds / 29 / 2;

}

FIGURE: CIRCUIT

FIGURE: CIRCUIT WITH BUZZER

Learning outcomes (What I have learnt):

- The furthest we can record is 1197 cm.

- We put conditions such that when the distance is less than 50 cm it will blink the led at PORT number 13.

- Another observation we have noticed is by attaching a buzzer with above condition we can make a security device which will start ringing whenever something approaches closer than 50 cm.

Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |